RISAT-1 SigmaSAR V3.1  -User Manual

1. Overview

SigmaSAR software is  developed at Microwave  Remote Sensing and Data Processing Division, MRSDPD/MRS&GDPG/DPPA&WAA/NRSC to generate Sigma Naught  and Gamma Naught Backscatter Images for Level-2 GeoTIFF Level-1 Ground Range and  Level-1 Slant Range data products of FRS-1 ,FRS-2, MRS and CRS  acquisition modes of  RISAT-1 .

The software can read the RISAT-1 CEOS format of Level-1 Data and GeoTIFF format of Level-2 and Level-2A Enhanced Geo-Referenced datasets.

Despeckling algorithms suitable for RISAT-1 data like Lee-Sigma and Gamma MAP have been implemented. The filtered images along with the Despeckled Sigma Naught products   for the various polarizations are   provided as the outputs.

2. Installation Guidelines

The software is a GUI-based application for Windows 32-bit and 64- bit architecture.

3. How to Install in Windows

3.1. Separate executables are available for 32-bit and 64-bit systems and the corresponding zip folder can be downloaded and unzipped. Microsoft dotNET 4.0  installer is also provided along with the application    .

3.2. Navigate to the folder “SigmaSARV3_64bit”and click on the “APPGUI_64” application and the GUI application opens up as shown below:

Figure 1: SigmaSAR GUI Application

3.3. The Sigma Naught products without and with de-speckle algorithms with 3X3 and 5X5 kernel sizes   can be generated using the corresponding Button Controls. The Gamma Naught Products can also be generated for various processing levels.

3.4. On selecting one of the Button controls, the   folder browser dialog box opens up and the input data folder may be chosen. Care must be taken to select the right input vis-à-vis Level-2 /Level-1 Input folder for the corresponding Button Control.

The Help Button may be clicked to view the User Manual   and the Exit Button may be used to   Exit the Application.

3.5. The RISAT-1 Level-2  and Level-2A data structure  are  as shown in Figure 2a and Figure 2b .

Figure 2a: RISAT-1 Data Format Structure for Level-2

Figure 2b: RISAT-1 Data Format Structure for Level-2A-Enhanced

3.6. The RISAT-1 Ground Range (MLG) and Slant Range (SLC) data structure is as shown below:

Figure 3a: RISAT-1 Data Format Structure for Level-1 MLG (Ground Range)

Figure 3b: RISAT-1 Data Format Structure for Level-1 SLC (Slant Range)

3.7. When the workorder folder is given as the input, the software fetches  further files for processing .The Sigma Naught and Gamma Naught Products are generated in the same input folder in the GeoTIFF format for Level-2 and Level-2A and in TIFF format for Level-1 data.

3.8. For Windows-32 bit systems, navigate to the folder “SigmaSARV3_32bit” and click on the “APPGUI_32” application. Microsoft dotNET 4.0  installer is also provided along with the application    .

3.9.The background console application may throw a few Error and Warning Messages regarding   TIFF format writing which may be ignored.  The  application may throw exceptions  if  the input  workorder  folder  does not contain all the required files as presented in the Data structure  as shown in  Figures 2a ,2b,3a and 3b. In order for the application to run smoothly it is advised to Exit and again restart the APPGUI   after every processing.

For Level-1 Single Look Complex Products, despeckling is done after detection   only and further upgradations will be implemented   and uploaded soon.

SigmaSAR V3.1 has been developed using C/C++ with .NET controls and is provided with the required libraries and executables. NRSC reserves the right to   modify and enhance the software as and when requirements arise. This is Beta Version    and   continual improvements  would be incorporated in the  application.

All user queries will be answered by the designer and suggestions   and feedback are welcome for further enhancement of the Application.

For queries and feedback please contact haripriya_s@nrsc.gov.in, sitakumari_evs @nrsc.gov.in, prasad_avv@nrsc.gov.in.